Procurement of server farm for the needs of the social sector

ICB No: SPAP II G-2-1-1/17

CLARIFICATION 8

- The network factory should have the ability to operate differently at different geographical sites[DZ1] .

What exactly is meant by operate differently?

It means ability to be configured as separate system.

- It should support a mode of operation in which the network switching factory represents a single factory with switches located at different sites. It should also support a mode in which the data transmission at different sites is separated, but the management is implemented by a single controller[DZ2] .

The sentence should contain a controller for fabric?

It depends on provided solution.

- The network factory should have a management and monitoring interface [DZ3] implemented in failover mode on a dedicated server or module. Network factory management modules or servers should not affect data transmission, even if they are unavailable or disabled.

Are you talking about a management system?

This requirement is for network management schema.

2.1 	Requirements for the Organization of Network Factory Topology of Each Site

- Communication between sites can be performed through separate switches providing inter-site communication.[DZ4]

Please describe the connection scheme between the sites. How many such sites. How exactly is the connection between sites organized?

Do I need to lay individual switches as border leaves, except for the two leaves below?

The other sites are not a scope for this bidding, however provided solution must support multiple site connection.

- 2.3.1 Data Center Spine Switch Requirements

| Productivity | Bandwidth: at least 1.6 Tb/s, The system should provide fast port-to-port processing time with a latency of no more than 350ns,[DZ5] System memory: at least 8GB, Solid State Disk (SSD) Memory: at least 16GB |
| --- | --- |

What applications require 350ns?

A very small value for a switch. We suggest to increase to 900ns.

The requirement is mandatory. There are many manufacturers with required performance.

| Data transmission | Switches should support data transmission using the VXLAN [DZ6] protocol according to the above requirements. At least 16 ports 100GbE per link aggregation group. Multi-chassis LAG for active/active L2 multipathing |
| --- | --- |

Static or EVPN?

This is not mentioned in bid. Suppliers should design the solution itself.

- 2.3.2  Data Center Leaf Switch Requirements

| Productivity | Bandwidth: at least 800 Gb/s, The system should provide fast port-to-port processing time with a latency of no more than 350ns,[DZ7] System memory: at least 8GB, Solid State Disk (SSD) Memory: at least 16GB |
| --- | --- |

What applications require 350ns?

A very small value for a switch. We suggest to increase to 900ns.

The requirement is mandatory. There are many manufacturers with required performance.

| Data transmission | Switches should support data transmission using the VXLAN [DZ8] protocol according to the above requirements. At least 20 ports 1/10/25/100GbE per link aggregation group. Multi-chassis LAG for active/active L2 multipathing |
| --- | --- |

EVPN/VXLAN l3 gateway?

This is not mentioned in bid. Suppliers should design the solution itself.

# 7. NETWORK SWITCHES

- The system should provide fast port-to-port processing time, the latency of which should be no more than 350ns.[DZ9]

What services need to provide 350ns?

The requirement is mandatory. There are many manufacturers with required performance.

NETWORK SECURITY FIREWALL

- Additional interface modules should provide ports with the following network [DZ10] connection speeds: at least 1 Gbit/s, 10 Gbit/s, and 40 Gbit/s.

Why do additional interfaces have to be mandatory on an additional interface module?

We can offer firewalls that fit the requirements and immediately have 1/10/40/100G interfaces.

The requirement is mandatory. There are many manufacturers with required performance.

The device should have the ability to add at least 4 interfaces with a bandwidth of not less than 10G, which will ensure data transfer without inspection in case of complete failure of the device. [DZ11]

What exactly are we talking about? Need to add an optical passive bypass? Which should switch traffic when the firewall goes down?

But the requirements indicate two boxes that should work in Active-Standby mode

Offer with passive bypass will be accepted.

- Ability to create and manage SD-WAN network.[DZ12]

What exactly is required from a network firewall?

All requirements for network firewall are described in chapter 9. NETWORK SECURITY FIREWALL

- ROUTERS

Please clarify the positioning of these devices. By request, this is a mixture of firewall (support for different VPN, Firewall, NAT, and a router with a large RIB and VRF

| Storage | Software storage. Duplicate OS instances on the same Route Processor[DZ13] . NSF/SSO capability. |
| --- | --- |

What exactly is required?

| VPN services | IPsec, GRE + IPsec, VTI[DZ14] , FlexVPN, Easy VPN, Enhanced Easy VPN, Dynamic Multipoint VPN (DMVPN), 2547 over DMVPN, [DZ15] Group Encrypted Transport VPN (GET VPN), IPv6 IPsec SVTI: [DZ16] |
| --- | --- |

Cisco proprietary term. We suggest fixing it for VTI or Equivalent.

Cisco proprietary technologies. Please remove from requirements. FlexVPN, Easy VPN, Enhanced Easy VPN, Dynamic Multipoint VPN (DMVPN), 2547 over DMVPN

Cisco proprietary technology. Please remove from IPv6 requirements IPsec SVTI

| Network Foundation Protection | ACL, FPM[DZ17] , Control Plane Protection (CPPr), Control Plane Policing (CoPP), QoS, Rolebased CLI Access, Sourcebased RTBH, uRPF, SSHv[DZ18] . |
| --- | --- |

What is FPM? Cisco Flexible Packet Matching?

What exactly is required?

What is SSHv?

| IPv4 Routing protocols | RIP v1/v2, EIGRP[DZ19] , OSPF, BGP, PBR Supports up to 1 million IPv4 routes BGP RR Scalability [DZ20] up to 5,250,000 IPv4 or 4,250,000 IPv6 routes.[DZ21] |
| --- | --- |

EIGRP is a Cisco proprietary protocol. Please remove from requirements.

According to the requirements, this is a full-fledged Firewall, not a Router.

It is wrong to use Firewall as RR. In theory, a router for these purposes is also not very good to use. The best solution for RR is virtual RR.

Where can so many routes come from, especially 4+ million IPv6 routes? 4,250,000 IPv6 routes

| VRF | Scales not less than 8,000 Virtual Route Forwarding (VRF) instances.[DZ22] |
| --- | --- |

Please clarify why 8000 VRF is required on such a device.

| Netflow v5/v9 | Yes: Netflow Event Logging— For Firewall and NAT services[DZ23] : yes. |
| --- | --- |

Should there be NAT support?

| CCP | WCCPv2 WCCP aware of VRF.[DZ24] |
| --- | --- |

# 1.         	Cisco proprietary technology. Please remove from requirements.

All requirements for router are described in chapter 11. Routers

The router can be from any manufacturer with minimum parameters similar to the characteristics required in the document. There shall not be any vendor-specific requirements.

- LAN AVAILABILITY SWITCHES

38.       	At least 48 x 10/100/1000Mbps BASE-T PoE+ (with up to 60W PoE capability per port[DZ25] ) inputs

Please clarify what equipment is planned to be connected to these switches? Are 60W PoE required on all ports? In this case, the PoE budget will be about 3KW - this is a lot.

Not all ports will work at .bt technology, however all ports should have the ability to provide 60w power.

·           	L2 capabilities: IEEE802.1ad QinQ VLAN, VLAN tagging capability IEEE 802.1Q (4094 VLAN IDs) and 2K VLANs simultaneous Jumbo packets, frame size up to 9220 bytes IEEE 802.1v STP, PVST+, Rapid Per-VLAN Spanning Tree [DZ26] GVRP, MVRP VxLAN encapsulation (tunneling) protocol.

Cisco proprietary protocol. We propose to reformulate the requirement as “PVST+ or VSTP”

The switch can be from any manufacturer with minimum parameters similar to the characteristics required in the document. There shall not be any vendor-specific requirements.

Security: IEEE 802.1X, TPM-based Security[DZ27] , ACL, Control plane policing, HTTPS, Port Security, BPDU guard, DHCPguard, STP root guard, IEEE 802.1AE MACsec

Why is TPM required on a LAN switch.

This is not a scope of discussion. Requirements are mandatory.