| Architecture | ARM 32bit |
| --- | --- |
| CPU | IPQ-8064 |
| CPU core count | 2 |
| CPU nominal frequency | 1.4 GHz |
| Dimensions | 443x92x44mm |
| License level | 5 |
| Operating System | RouterOS |
| Size of RAM | 1 GB |
| Storage size | 128 MB |
| Storage type | NAND |
| Tested ambient temperature | -20°C to 70°C |
| Suggested price | $179.00 |

## Powering

| Details |  |
| --- | --- |
| PoE in | Passive PoE |
| PoE in input Voltage | 10-30 V |
| PoE out | Passive PoE |
| PoE-out ports | Ether10 |
| Number of DC inputs | 2 (DC jack, PoE-IN) |
| DC jack input Voltage | 10-30 V |
| Max out per port output (input < 30 V) | 600 mA |
| Max total out (A) | 600 mA |
| Max power consumption | 30 W |
| Max power consumption without attachments | 10 W |

## Ethernet

| Details |  |
| --- | --- |
| 10/100/1000 Ethernet ports | 10 |

## Fiber

| Details |  |
| --- | --- |
| SFP ports | 1 |

## Peripherals

| Details |  |
| --- | --- |
| Serial port | RJ45 |
| Number of USB ports | 1 |
| USB Power Reset | Yes |
| USB slot type | USB 3.0 type A |
| Max USB current (A) | 1 |

## Other

| Details |  |
| --- | --- |
| PCB temperature monitor | Yes |
| Voltage Monitor | Yes |