Dimensions 420 x 420 x 275 mm (16.54 x 16.54 x 10.83")

Weight 1.753 kg (3.87 lb)

Power Supply 24V, 0.5A Gigabit PoE Max.

Power Consumption  8W Operating

Frequency Worldwide USA: U-NII-1 USA: U-NII-2A USA: U-NII-2C USA: U-NII-3 5150 - 5875 MHz 5150 - 5250 MHz* 5250 - 5350 MHz* 5470 - 5725 MHz* 5725 - 5850 MHz*

Gain 25 dBi

Networking Interface (1) 10/100/1000 Ethernet Port

Processor Specs  Atheros MIPS 74Kc, 560 MHz

Memory 64 MB DDR2, 8 MB Flash

LEDs (1) Power, (1) LAN, (4) WLAN

Signal Strength LEDs Software-Adjustable to Correspond to Custom RSSI Levels

Max. VSWR 1.5:1

Channel Sizes 5/8/10/20/30/40 MHz

Polarization Dual Linear

Enclosure Outdoor UV Stabilized Plastic

Mounting Pole-Mount (Kit Included)

Wind Loading 278.4 N @ 120 km/h (63 lbf @ 75 mph)

Wind Survivability 120 km/h (75 mph)

ESD/EMP Protection Air: ± 24 kV, Contact: ± 24 kV

Operating Temperature -40 to 70° C (-40 to 158° F)

Operating Humidity 5 to 95% Noncondensing

Wireless Approvals FCC, IC, CE

RoHS Compliance Yes

Salt Fog Test IEC 68-2-11 (ASTM B117), Equivalent: MIL-STD-810 G Method 509.5

Vibration Test IEC 68-2-6

Temperature Shock Test IEC 68-2-14

UV Test IEC 68-2-5 at 40° C (104° F), Equivalent: ETS 300 019-1-4

Wind-Driven Rain Test ETS 300 019-1-4, Equivalent: MIL-STD-810 G Method 506.5