| Sensor | 1/3″, 4.0 megapixel, progressive scan, CMOS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lens | 4.0mm@F2.0 |  |  |  |  |  |  |  |
| DORI Distance | Lens | Detect(m) |  | Observe(m) |  | Recognize(m) |  | Identify(m) |
|  | 4.0mm | 60 |  | 24 |  | 12 |  | 6 |
|  | 6.0mm | 90 |  | 36 |  | 18 |  | 9 |
| Angle of View (H) | 78.9° |  |  |  |  |  |  |  |
| Angle of View (V) | 39.6° |  |  |  |  |  |  |  |
| Angle of View (O) | 97.0° |  |  |  |  |  |  |  |
| Adjustment angle | Pan: 0°~360° |  | Tilt: 0°~90° |  |  |  | Rotate: 0°~360° |  |
| Shutter | Auto/Manual, 1~1/100000 s |  |  |  |  |  |  |  |
| Minimum Illumination | Colour: 0.03 Lux (F2.0, AGC ON) 0 Lux with IR on |  |  |  |  |  |  |  |
| Digital noise reduction | 2D/3D DNR |  |  |  |  |  |  |  |
| Day/Night | IR-cut filter with auto switch (ICR) |  |  |  |  |  |  |  |
| S/N | >52dB |  |  |  |  |  |  |  |
| IR Range | Up to 30m (98 ft) IR range |  |  |  |  |  |  |  |
| WDR | DWDR |  |  |  |  |  |  |  |
| Video |  |  |  |  |  |  |  |  |
| Video Compression | Ultra 265,H.265, H.264, MJPEG |  |  |  |  |  |  |  |
| H.264 code profile | Baseline profile, Main Profile |  |  |  |  |  |  |  |
| Frame Rate | Main Stream:4MP (2592*1520): Max. 20 fps; Sub Stream:720P (1280*720): Max. 20 fps; Third Stream:D1 (720×576): Max. 20 fps |  |  |  |  |  |  |  |
| HLC | Supported |  |  |  |  |  |  |  |
| BLC | Supported |  |  |  |  |  |  |  |
| OSD | Up to 4 OSDs |  |  |  |  |  |  |  |
| Privacy Mask | Up to 8 areas |  |  |  |  |  |  |  |
| ROI | Up to 2 areas |  |  |  |  |  |  |  |
| Motion Detection | Up to 4 areas |  |  |  |  |  |  |  |
| Network |  |  |  |  |  |  |  |  |
| Protocols | IPv4,IGMP,ICMP,ARP,TCP,UDP,DHCP, RTP,RTSP,RTCP,DNS,DDNS,NTP,FTP,UPnP,HTTP,HTTPS,SMTP, SSL |  |  |  |  |  |  |  |
| Compatible Integration | ONVIF(Profile S, Profile T), API |  |  |  |  |  |  |  |
| Interface |  |  |  |  |  |  |  |  |
| Network | 10/100M Base-TX Ethernet |  |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |  |
| Power | 12 V DC±25%, PoE (IEEE802.3 af) |  |  |  |  |  |  |  |
|  | Power consumption: Max 4W |  |  |  |  |  |  |  |
| Dimensions (L × W × H) | 167.3 × 62.9 ×62.7 mm (6.6” × 2.5” × 2.5”) |  |  |  |  |  |  |  |
| Weight | 0.37kg (0.81lb) |  |  |  |  |  |  |  |
| Working Environment | -30°C ~ +60°C (-22°F ~ 140°F), Humidity :10%~90% RH(non-condensing) |  |  |  |  |  |  |  |
| Ingress Protection | IP67 |  |  |  |  |  |  |  |