| IPC2325LBR3-SP-D |  |  |  | IPC2325LBR3-SPZ28-D |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Camera |  |  |  |  |  |  |  |
| Sensor | 1/2.7″, 5.0 megapixel, progressive scan, CMOS |  |  |  |  |  |  |
| Lens | 2.8~12 mm, motorized zoom lens |  |  |  |  |  |  |
| DORI Distance | Lens | Detect(m) |  | Observe(m) | Recognize(m) |  | Identify(m) |
|  | 2.8mm | 63 |  | 25.2 | 12.6 |  | 6.3 |
|  | 12mm | 270 |  | 108 | 54 |  | 27 |
| Angle of View (H) | 105.3°~25.4° |  |  |  |  |  |  |
| Angle of View (V) | 69.5°~18.4° |  |  |  |  |  |  |
| Angle of View (O) | 136.9°~31.2° |  |  |  |  |  |  |
| Adjustment angle | Pan: 0°~360° |  | Tilt: 0°~90° |  |  | Rotate: 0°~360° |  |
| Shutter | Auto/Manual, 1~1/100000 s |  |  |  |  |  |  |
| Minimum Illumination | Colour: 0.01 Lux (F1.8, AGC ON) 0 Lux with IR on |  |  |  |  |  |  |
| Digital noise reduction | 2D/3D DNR |  |  |  |  |  |  |
| Day/Night | IR-cut filter with auto switch (ICR) |  |  |  |  |  |  |
| S/N | >52dB |  |  |  |  |  |  |
| IR Range | Up to 30m (98 ft) IR range |  |  |  |  |  |  |
| WDR | DWDR |  |  |  |  |  |  |
| Video |  |  |  |  |  |  |  |
| Video Compression | Ultra 265,H.265, H.264, MJPEG |  |  |  |  |  |  |
| H.264 code profile | Baseline profile, Main Profile |  |  |  |  |  |  |
| Frame Rate | Main Stream:5MP (2592*1944): Max. 20fps; Sub Stream: 720P (1280*720): Max. 20 fps; |  |  |  |  |  |  |
| HLC | Supported |  |  |  |  |  |  |
| BLC | Supported |  |  |  |  |  |  |
| OSD | Up to 4 OSDs |  |  |  |  |  |  |
| Privacy Mask | Up to 4 areas |  |  |  |  |  |  |
| ROI | Up to 2 areas |  |  |  |  |  |  |
| Motion Detection | Up to 4 areas |  |  |  |  |  |  |
| Network |  |  |  |  |  |  |  |
| Protocols | IPv4, IGMP, ICMP, ARP, TCP, UDP, DHCP, RTP, RTSP, RTCP, DNS, DDNS, NTP, FTP, UPnP, HTTP, HTTPS, SMTP, SSL |  |  |  |  |  |  |
| Compatible Integration | ONVIF(Profile S, Profile G, Profile T), API |  |  |  |  |  |  |
| Interface |  |  |  |  |  |  |  |
| Network | 1 RJ45 10M/100M Base-TX Ethernet |  |  |  |  |  |  |
| Edge Storage | MicroSD, up to 256GB |  |  |  |  |  |  |
| General |  |  |  |  |  |  |  |
| Power | 12 V DC±25%, PoE (IEEE802.3 af) |  |  |  |  |  |  |
|  | Power consumption: Max 7.0W |  |  |  |  |  |  |
| Dimensions (L × W × H) | 240.3×86.4×75.6mm(9.46”×3.4”×2.98”) |  |  |  |  |  |  |
| Weight | 1kg（2.2lb） |  |  |  |  |  |  |
| Working Environment | -30°C ~ +60°C (-22°F ~ 140°F), Humidity :10%~90% RH(non-condensing) |  |  |  |  |  |  |
| Ingress Protection | IP67 |  |  |  |  |  |  |